PLOT PROGRAM

Copyright 1995 by Brian Beezley, K6STI

All Rights Reserved

PLOT Program...................................1

Running PLOT...................................1

Polar Plots....................................2

Rectangular Plots..............................2

Overlaying Plots...............................2

Comparing Plots................................3

Screen Colors..................................3

Printing The Screen............................3

Graphics Image Files...........................3

SET Commands...................................4

Index..........................................8

---- PLOT PROGRAM ----------------------------------------------

PLOT.EXE displays and prints far-field antenna radiation

patterns generated by the MN Antenna Analysis program, the AO

Antenna Optimizer, the YO Yagi Optimizer, NEC/Yagis, or NEC/

Wires.  PLOT requires a 386+387 or better, VGA, and DOS 3.0 or

later.

If PLOT won't run on your computer, use the PLOX

plotting program instead.  (Some of the features described below

aren't implemented in PLOX and others are implemented in a

different way.  Press F1 for help in PLOX.)

---- RUNNING PLOT ----------------------------------------------

The quickest way to display a plot is to give the name

of a plot file on the command line, for example, PLOT DIPOLE

(the .PLT file extension isn't required).  If you don't know

which file you want, type PLOT and plot files in the current

directory will be listed.

Select a file by moving the lightbar with the arrow

keys, PgUp, PgDn, Home, or End.  Press Enter to select the

highlighted file.  Alternatively, you can type a filename.  As

you type, the lightbar moves to the first filename that matches

the characters entered.  Press Enter whenever the desired file

is highlighted.

Select the last item ("Other") to enter a file or

directory name not listed.  If you enter a directory containing

plot files, PLOT lists them and you can select one.  See the SET

Commands section for information on organizing plot files into

subdirectories.

PLOT normally begins by drawing a polar plot of the

azimuth pattern using the log-dB scale.  You can start PLOT in

other modes by supplying up to three option letters after the

filename as follows:

E       Elevation pattern

R       Rectangular plot

L       Linear-dB scale

For example, PLOT DIPOLE ERL will cause PLOT to draw the

elevation pattern in rectangular coordinates.  If you later

select a polar plot, it will use the linear-dB scale.

Press F1 or any other nonfunctional key for help.  The

help panel is context-sensitive; keys that aren't currently

active are gray.  For example, the X and Y keys are gray in

polar mode.

1

---- POLAR PLOTS -----------------------------------------------

Select a polar plot with the P key.  You can use one of

two radial scales.  The standard ARRL log-dB scale causes lower-

level sidelobes to be compressed toward the center of the

pattern.  This emphasizes the shape of the major lobe.  The ARRL

log-dB scale is widely used in amateur publications so it yields

patterns with familiar shapes.  The center of the plot is minus

infinity dB (no signal), but there is little plot area below -40

dB.

The other radial scale uses linear dB and cuts off at

-50 dB at the center of the plot.  This scale provides much more

area between -20 and -50 dB than the log-dB scale.  It's useful

for examining low-level sidelobes that may be hard to see in a

log-dB plot.

The dots forming the radial lines (the ones that aren't

multiples of 30 degrees) are spaced every 2 dB.  The dots

forming the circles are spaced at multiples of 1 degree.  These

calibrations allow you to read directivity values from the plots

with good accuracy.

The polar plots are perfectly circular on monitors with

standard 4:3 aspect ratio.  If the plots appear elliptical,

adjust your monitor's vertical height control.

---- RECTANGULAR PLOTS -----------------------------------------

Use the R key to change to rectangular coordinates.  A

rectangular plot can reveal small sidelobe detail even better

than a linear-dB polar plot, but the overall pattern shape is

not quite as apparent.  The X axis of a rectangular plot is

azimuth or elevation angle.  The Y axis is antenna response in

dB using a linear scale.  Use the Y key to change the Y-axis

cutoff (lower limit in dB).  This parameter is always a negative

number, but you can enter it without a minus sign for

convenience.  The Y-axis cutoff must be between -1 and -100 dB.

The Y axis is easiest to interpret when the cutoff value is a

multiple of 10 degrees, but you can enter any value and the

scale and grid will be drawn correctly.  Use the X key to change

the X-axis cutoff.  The value you enter will be rounded to the

nearest multiple of 10 degrees within the range of 10 to 180

degrees.

---- OVERLAYING PLOTS ------------------------------------------

Use the O key to overlay a second pattern in a different

color.  Plot annotations use the same color as the corresponding

plot traces.  Filenames replace plot titles for overlayed plots.

2

---- COMPARING PLOTS -------------------------------------------

Use the C key to select a second plot file for pattern

comparison.  The new plot is drawn into a separate screen

buffer.  Use the spacebar to instantly switch buffers.  This

gives a precise, instantaneous, pattern comparison that can

reveal differences obscured by screen clutter when plots are

overlayed.

Use the N key to normalize compared patterns to the same

scale.  Press N again to revert to full-scale pattern peaks.

---- SCREEN COLORS ---------------------------------------------

F5 changes screen colors.  You can change colors for the

file screen and for the pattern screen.  Use the left/right

arrows to select a screen item.  Then use Home/End, up/down

arrows, and PgUp/PgDn to cycle through 64 intensities for each

of the red, green, and blue color components.  This yields 256K

color choices.  After selecting colors, save them by pressing S.

This writes color codes to the PLOT.INI file.  PLOT reads

PLOT.INI whenever it begins execution and sets the colors

accordingly.  If PLOT.INI isn't present, PLOT uses a default

color set.  You can delete PLOT.INI to return to the default

color set.

---- PRINTING THE SCREEN ---------------------------------------

PLOT prints text and graphics screens to HP LaserJet/

DeskJet printers and to Epson-compatible, dot-matrix printers.

Press F10 to print any screen.  See the SET Commands section for

details on configuring PLOT for your particular printer.  Unless

you're in LaserJet landscape mode, PLOT does not eject the page

after printing a screen.  This lets you print two screens on one

page.

The PSPLOT utility generates gorgeous plots on

PostScript printers from plot files.  PSPLOT can overlay

patterns and can do multiple plots per page.  PSPLOT is

available for $25 from Paul Terwilliger, NX1H, 11 North Shore

Rd., Chester, NH 03036, (603) 887-4346.

---- GRAPHICS IMAGE FILES --------------------------------------

PLOT can save the image of any graphics screen in the

.PCX file format.  This feature lets you add PLOT graphics to

desktop-publishing and word-processing documents.  If your

computer has a fax/modem, you can fax PLOT images.

Press F9 to generate a .PCX file.  The output filename

is the antenna filename with the extension .PCX.  Whenever you

press F9 again, an incrementing digit is appended to the

filename and another file is created.  You can control image

3

size and centering with a DOS SET command.  See the next section

for details.

---- SET COMMANDS ----------------------------------------------

DOS provides a convenient way to specify configuration

information to PLOT.  The DOS SET command places information you

supply into the DOS environment in memory where it can be

retrieved later by a program.  It's convenient to put SET

commands in your AUTOEXEC.BAT file so they'll be executed

automatically every time the computer boots.  You can enter SET

commands in upper or lower case.  It's best not to leave spaces

around the = in SET commands.

You can see what's in the DOS environment by typing the

following:

SET

You can eliminate a SET parameter P by typing this:

SET P=

If you use lots of SET commands, it's possible to run

out of environment space.  To enlarge it to 1024 bytes, put a

line similar to this one in your CONFIG.SYS file:

SHELL=\COMMAND.COM /E:1024 /P

1.  Subdirectories

Once you accumulate many plot files, it's nice to

organize them into subdirectories.  You might use the current

directory for antenna experiments, saving optimized plots

elsewhere.  You can tell PLOT to automatically reference certain

subdirectories with a SET command.

Define subdirectories like this:

SET PLT=. PLOTS1 PLOTS2 PLOTS3

The period represents the current directory.  It may

appear anywhere (or nowhere) in the list.  You can define as

many subdirectories as you like.  You can organize

subdirectories by antenna type, frequency range, designer, etc.

Without a SET command, the subdirectory list consists of

just the current directory.

PLOT begins by listing files in the first subdirectory

on the list.  Subdirectory names are listed after the files.

Select a subdirectory to list its files.

4

You can specify a plot file from one of the

subdirectories on the PLOT command line without typing the path.

PLOT searches the subdirectories in the order given in the

subdirectory list.

You can specify any subdirectory (not necessarily from

the list) on the command line and PLOT will list its plot files.

2.  Gain Reference

PLOT displays gain figures in dBi by default.  Gain in

dBi is antenna response compared with that of an isotropic

radiator in free space.  An isotropic antenna radiates uniformly

in all directions.  To compare antenna response with that

broadside to a free-space, half-wave dipole, do the following:

SET DB=dBd

Gain figures are 2.15 dB lower when expressed in dBd.

3.  Printer Type

To print graphics screens on many 9-pin, dot-matrix

printers, no SET command is required.  However, some printers do

not recognize the special line-spacing command PLOT uses to

print an exact screen image.  If your 9-pin printer won't print

PLOT graphics, try the following:

SET PINS=9ALT

Even with 9ALT, PLOT may be unable to print to certain

old 9-pin printers.

For a 24-pin, dot-matrix printer, use:

SET PINS=24

For an HP LaserJet or DeskJet printer, use:

SET PINS=HPLJ

To try a landscape plot (not available on all laser

printers), use:

SET PINS=HPLJ L

For a bigger landscape plot, use:

SET PINS=HPLJ LB

PLOT normally draws LaserJet/DeskJet plots with a

border.  To eliminate the border, add an X like this:

SET PINS=HPLJ X or LX or LBX

5

4.  Printer Port

Graphics screens print on LPT1 by default.  To print

graphics on another port, use one of the following:

SET LPT=LPT2

SET LPT=LPT3

5.  .PCX Output

By default, PLOT generates .PCX screen images with 640

by 480 pixels.  These images can be directly incorporated in

documents.  However, the images won't be centered on fax pages

because they have no margins.  You can add margins with the

following command:

SET PCX=left top bottom

The three numbers specify margin size in inches.  The

bottom margin is optional.  You can use it to pad the image to

form a complete page if your fax software doesn't do this

automatically.  Try left = .8 and top = 2.35 for 8.5" by 11"

pages.

You can create a double-size .PCX image by adding the

keyword BIG to the line (use upper or lower case).  BIG is for

high-resolution fax mode.

6.  Display Typeface

You can use the standard serif typeface built into your

VGA card with the following:

SET TYPEFACE=SERIF

7.  VGA Compatibility

PLOT can use a dual-paging scheme to provide smoother

graphics by fully buffering all screen changes.  This scheme

requires a compatible VGA card with 512K video memory.  If your

card is incompatible or has less memory, you'll get extraneous

graphics images.  Dual paging eliminates the blink when you

change to a new screen.  To try it, do the following:

SET PAGES=2

PLOT programs the VGA overscan register to extend the

background color into the overscan region surrounding the active

screen.  This improves the appearance of text at screen edges.

However, some VGA cards (for example, those using the Tseng Labs

ET4000 VGA chip) generate the wrong overscan color.  This causes

6

a distinct, off-color border.  To fix this problem, do the

following:

SET OVERSCAN=FIX

8.  Screen Bounce

When switched from graphics to text mode, the screens of

many monitors bounce or break up during resynchronization.  PLOT

can blank the screen for a short period while the monitor

settles.  To try this, specify a short delay (like 0.1) in

seconds as follows:

SET BOUNCE=delay

7

INDEX

.PCX file  3

24-pin  5

9-pin  5

Annotations  2

ARRL log-dB scale  2

Arrow keys  1

Aspect ratio  2

AUTOEXEC.BAT  4

BIG  6

Blink  6

Border  5

Calibrations  2

Circles  2

Color  2, 3

Color codes  3

Command line  1

COMMAND.COM  4

Comparing Plots  3

CONFIG.SYS  4

Current directory  1, 4

DBd  5

DBi  5

Default color set  3

DeskJet  3, 5

Directivity values  2

DOS environment  4

Dot-matrix printers  3, 5

Dots  2

Dual-paging scheme  6

Eject  3

Elliptical  2

End  1

Enter  1

Epson-compatible  3

Extraneous graphics images  6

F1  1

F10  3

F5  3

F9  3

Fax  3

Gain Reference  5

Graphics Image Files  3

Graphics screens  6

Help panel  1

Highlight  1

8

Home  1

HP LaserJet  3, 5

Isotropic antenna  5

Landscape plot  5

Lightbar  1

Linear dB  2

Linear-dB scale  1

List  1

Log-dB scale  1

Lower-level sidelobes  2

LPT  6

Major lobe  2

NX1H  3

Option letters  1

Other  1

Overlay  2, 3

Path  5

Pattern comparison  3

PCX file  3

PCX Output  6

PgDn  1

PgUp  1

PLOT.INI  3

PLOX  1

Polar Plots  2

PostScript  3

Print  3

Print graphics  6

Printer Port  6

Printer Type  5

Printing The Screen  3

PSPLOT  3

Radial lines  2

Radial scales  2

Rectangular coordinates  1

Rectangular Plots  2

Rounded  2

Running PLOT  1

Save  3

Screen Bounce  7

Screen buffer  3

Screen clutter  3

Screen Colors  3

Select a file  1

Serif typeface  6

SET BOUNCE  7

SET DB  5

SET LPT  6

9

SET OVERSCAN  7

SET PAGES  6

SET PINS  5

SET PLT  4

SET TYPEFACE  6

SHELL  4

Sidelobe detail  2

Smoother graphics  6

Spacebar  3

Subdirectories  4

Terwilliger  3

Typeface  6

Upper or lower case  4

Vertical height control  2

VGA Compatibility  6

Video memory  6

X-axis cutoff  2

Y-axis cutoff  2

10